Introduction:

BPQAXIP.DLL supports the encapsulation of AX.25 frames

in IP, as defined in RFC1226.

It also supports UDP encapuslation, as used by some UNIX systems.

Environment.

The program uses Winsock Version 2. This is provided

as standard in NT 4 and Windows 98, but must be installed

on Win 95 before the program can be used. The Winsock2 for 95

runtime should be included with this program - if not, you

will need to download it from Microsoft (ws2setup.exe).

The driver is loaded when BPQ32 initialises the port.

This occurs when the first program to use the DLL is

loaded. Other BPQ32 programs may be loaded and unloaded,

but if the first program exits before all the others,

the AXIP dll will be deleted, and the other programs

will crash. To get round this problem, I've written

a simple application (BPQAPP.EXE) which can be run first,

and does nothing but keep the switch and AXIP dlls in

memory. This program should be run before any other

programs that use the BPQ32 interface.

Configuration.

The driver is defined to BPQ32 as an External port:

PORT

ID=AXIP Link

TYPE=EXTERNAL

DLLNAME=BPQAXIP.DLL

QUALITY=200

MAXFRAME=4

FRACK=7000

RESPTIME=1000

RETRIES=10

PACLEN=120

ENDPORT

Note the DLLNAME parameter, which is new to BPQ32.

The DLL reads a configuration file BPQAXIP.CFG, which

defines the mapping from AX.25 callsign-ssid, to an IP

address.This file needs to be in the current directory, or the

directory pointed to by the registry entry

HKEY_LOCAL_MACHINE\Software\G8BPQ\BPQ32\Config File Location

when BPQ32 is loaded.

#	Config file for BPQAXIP

UDP 10093					# Optional. Enables UDP support, and defines the port

# AX.IP listens on

MAP G8BPQ-7 10.2.77.1		# Use Raw IP encapulation

MAP BPQ7	10.2.77.1

MAP NODES	10.2.77.1

MAP G8BPQ-7 10.2.77.1 UDP 10093	# Use UDP encapulation, send to port 10093

It is OK to mix Raw IP and UDP encapsulation, and to use a different

port for different destinations,

If an ssid is supplied, the entry only applies to that ssid value.

If no ssid is supplied, the entry applies to all ssid values.

So, if you want to select traffic for only G8BPQ, you must specify

G8BPQ-0.

The code has been modified to allow more that one mapping for each

callsign, so, for example NODES broadcasts can be sent to more than

one station. Unfortunately this means that it is no longer possible

to send traffic for G8BPQ to 10.2.77.1, and all other ssids to

10.2.77.2 by specifing:

MAP G8BPQ-0 10.2.77.1

MAP G8BPQ   10.2.77.2

If you try this, messages to G8BPQ will also be sent to 10.2.77.2

There is a limit of 128 entries in the mapping table.

The software at the moment only allows IP addresses -

it does not yet support lookup of host names.

The software will work with an IP broadcast address,

but if the destination is on the same net as the switch,

it will receive its own broadcast frames.

.

.